Technical Report: PiCar-X V2 Ball Follower Robot

Introduction

For this project, we designed and built an autonomous ball-following robot using the SunFounder PiCar-X V2 platform with a Raspberry Pi. The goal was to create a robot that can detect and follow a colored ball in real time by integrating sensors and actuators, demonstrating autonomous navigation and decision making.

(Sensor Fusion)

This report documents the hardware setup, software design, algorithm choices, testing outcomes, and the ethical considerations involved in making this robot operate safely and responsibly.

Hardware Setup

- The PiCar-X V2 robot kit provides a solid base for this project. It includes:

- A Raspberry Pi 5 as the main computer.

- A USB camera mounted on a pan-tilt servo mechanism for vision

- Two-wheel motors controlled by a driver board for movement

- An ultrasonic sensor for obstacle detection and avoidance

- Steering servo for direction control

All components are connected and controlled via the PiCar-X’s Python API. The pan-tilt camera allows the robot to actively track the ball as it moves, which provides a flexible and responsive system. The ultrasonic sensor adds a layer of safety, allowing the robot to avoid collisions by stopping for objects <20cm or rerouting when obstacles are detected further than 20 cm.

Software Design and Algorithms

The core of the ball-following capability relies on computer vision techniques for object detection paired with servo and motor controls.

First, the camera captures live video frames, which are processed using OpenCV to detect the ball. The approach involves converting each frame to HSV (Hue, Saturation, Value) color space and thresholding it based on calibrated color ranges matching the ball’s color (an White, green, black and red). This creates a mask highlighting regions likely to be the ball.

Next, contour detection locates the largest blob matching the color mask, and from it, the program calculates the center coordinates and size (radius) of the ball in the frame.

The servo controlling the camera’s pan angle is adjusted proportionally to the horizontal distance between the center of the frame and the ball’s detected position. This keeps the camera focused on the ball as it moves laterally. For movement, the robot checks the ball’s size in the frame (bigger means closer). If the ball is far, the robot moves forward slowly; if it is close enough, the robot stops.

This combination of real-time vision processing and sensor-based feedback allows the robot to follow a moving ball smoothly and halt safely when needed.

There are some concerning points for example the PiCar might not be able to track the ball if its elevation is beyond the cameras range. Another point is the hardware; the software of will be limited to the hardware capabilities. For the camera to react to sudden change in direction and speed of an object it will need a high-speed camera and potentially even a higher quality camera to keep up with the object, in our case the ball. Same applies to the PiCar’s movement, faster motors would mean it can get in range of the ball quicker so the camera could detect it.

Ethical Considerations

Safety was a primary consideration throughout the project. The robot’s speed is purposely kept low (despite already being limited by its hardware), and the ultrasonic sensor provides a mandatory stop function to avoid collisions. The visual data is transiently processed without storing images or personal data to ensure privacy including the live feed that’s displayed. Nothing that gets detected by the camera is recorded.

The system operates solely in controlled environments and does not perform any data sharing or decision making beyond the programmed behavior, minimizing risks.

As a student project, the robot’s deployment is limited to educational use. The code includes an emergency stop through software interruption (Ctrl+C) to allow immediate manual control.

This responsible design respects safety and privacy norms consistent with ethical robotics development.

Conclusion

This project successfully demonstrated the integration of real-time perception, autonomous navigation, and safe operation on the SunFounder PiCar-X V2 platform. The ball follower robot combines vision processing, servo control, motor actuation, and ultrasonic sensing into a functional and responsive system.

Challenges related to lighting and detection limitations provide interesting areas for future improvement, such as machine learning-based object recognition or adaptive lighting compensation.